CE211-L  Circuit Analysis Lab

Lab Report # 9

Submitted by: Shayan Rizwan Yazdanie [2024585]

Submitted to:  Engr. Muhammad Shakaib

Semester:  03

Faculty of Computer Science and Engineering

GIK Institute of Engineering Sciences and Technology

Task Statement:

- Calculate the current (IL) and voltage (Vo) across 1kΩ resistor in the given circuit.

Solution

## Calculating Thevenin Resistance RTH:

RTH=R1∥R2

RTH=R1⋅R2R1+R2
               =10k⋅2.2k10k+2.2k
    =22k212.2k
                                                                              =1803.279 Ω

## Calculating Thevenin Voltage VTH:

Using Voltage divider formula:

VTH=Vs(R2R1+R2)
                 =12(2.2k10k+2.2k)
    =12(2.212.2)
                                                           VTH=2.1639 V

## Calculating Load Current IL:

IL=VTHRTH+RL
IL=2.16391803.279+1000
IL=0.7719 mA

## Calculating Output Voltage Vo:

Vo=IL⋅RL
Vo=0.7719×10-3×1000
Vo=0.7719 V

## Final Answers:

- Load Current: IL=0.7719 mA

- Output Voltage: Vo=0.7719 V