CE221-L   Digital Logic Design Lab

Lab report # 08

Submitted by: Shayan Rizwan

Registration Number: 2024585

Submitted to:  Engr. Irfanullah

Semester: 03

Faculty of Computer Science and Engineering

GIK Institute of Engineering Sciences and Technology

Task Statement:

Design a full adder using Behavioral modeling in Verilog, write test benches to verify its functionality, and simulate to visualize the waveforms and show its RTL.

Solution

CODE:

module full_adder(a,b,c,carry,sum );

input a,b,c;

output sum,carry;

reg sum,carry;

always@( a or b or c)

begin

sum = a^b^c;

carry = (a&b) | c & (a^b);

end

endmodule

Test Bench:

` module full_adder_tb();

// Inputs

reg a;

reg b;

reg c;

// Outputs

wire sum;

wire carry;

full_adder uut(.sum(sum),.carry(carry),.a(a),.b(b),.c(c));

initial

begin

a = 0; b = 0; c=0;

#10 a = 0; b = 0; c=1;

#10 a = 0; b = 1; c=0;

#10 a = 0; b = 1; c=1;

#10 a = 1; b = 0; c=0;

#10 a = 1; b = 0; c=1;

#10 a = 1; b = 1; c=0;

#10 a = 1; b = 1; c=1;

$finish;

end

endmodule

RTL:

Output:

Task Statement:

Design a 4x1 Multiplexer using Behavioral modeling in Verilog, write test benches to verify its functionality, and simulate to visualize the waveforms and show its RTL.

Solution

Code

module mux_4x1(y,i,s);

output y;

input [3:0]i;

input [1:0]s;

reg y;

// Behavioural

always@(i or s)

// Using case statement

case(s)

3'b00:y = i[0];

3'b01:y = i[1];

3'b10:y = i[2];

3'b11:y = i[3];

default: y = 1'b0;

endcase

endmodule

Test Bench

module mux_4x1_tb();

// Inputs

reg [3:0] i;

reg [1:0] s;

// Outputs

wire y;

// Instantiate the Unit Under Test (UUT)

mux_8x1 uut ( .y(y),.i(i),.s(s));

initial

begin

// Initialize Inputs

i = 4'b1010;

s = 2'b00; #10;

s = 2'b01; #10;

s = 2'b10; #10;

s = 2'b11; #10;

i= 4'b1100;

s = 2'b00; #10;

s = 2'b01; #10;

s = 2'b10; #10;

s = 2'b11; #10;

i = 4'b0110;

s = 2'b00; #10;

s = 2'b01; #10;

s = 2'b10; #10;

s = 2'b11; #10;

i = 4'b1001;

s = 2'b00; #10;

s = 2'b01; #10;

s = 2'b10; #10;

s = 2'b11; #10;

i = 4'b1111;

s = 2'b00; #10;

s = 2'b01; #10;

s = 2'b10; #10;

s = 2'b11; #10;

i = 4'b0000;

s = 2'b00; #10;

s = 2'b01; #10;

s = 2'b10; #10;

s = 2'b11; #10;

$finish;

end

endmodule

RTL

Output

Task Statement:

Design a 3x8 Decoder using Behavioral modeling in Verilog, write test benches to verify its functionality, and simulate to visualize the waveforms and show its RTL.

Solution

CODE:

module decoder_3x8(input [2:0] a,output reg [7:0] out,input en);

always @(*)

begin

if (en)

case(a)

3'b000: out = 8'b00000001;

3'b001: out = 8'b00000010;

3'b010: out = 8'b00000100;

3'b011: out = 8'b00001000;

3'b100: out = 8'b00010000;

3'b101: out = 8'b00100000;

3'b110: out = 8'b01000000;

3'b111: out = 8'b10000000;

default: out = 8'b00000000;

endcase

else

out = 8'b00000000;

end

endmodule

Test Bench:

module deocoder_3x8_tb();

reg [2:0] a;

reg en;

wire [7:0] out;

decoder_3x8 uut(.a(a),.en(en),.out(out));

initial

begin

$monitor($time, " a=%b, en=%b, out=%b", a, en, out);

en = 0;

a = 3'b000; #10;

a = 3'b001; #10;

a = 3'b010; #10;

a = 3'b011; #10;

a = 3'b100; #10;

a = 3'b101; #10;

a = 3'b110; #10;

a = 3'b111; #10;

en = 1;

a = 3'b000; #10;

a = 3'b001; #10;

a = 3'b010; #10;

a = 3'b011; #10;

a = 3'b101; #10;

a = 3'b100; #10;

a = 3'b110; #10;

a = 3'b111; #10;

$finish;

end

endmodule

RTL:

Output:

Task Statement:

Design an ALU using Behavioral modeling in Verilog, write test benches to verify its functionality, and simulate to visualize the waveforms and show its RTL.

Solution

Code:

module ALU(input [8:0] a,input [8:0] b,input [3:0] sel,output reg [8:0] c);

always @(*)

begin

case(sel)

4'b0000: c = a+b;

4'b0001: c = a-b;

4'b0010: c = a*b;

4'b0011: c = a&&b;

4'b0100: c = a||b;

4'b0101: c = a&b;

4'b0100: c = a|b;

4'b0111: c = a<<1;

4'b1000: c = a>>b;

4'b1001: c = ~a;

4'b1010: c = !a;

4'b1011: c = a+1;

4'b1100: c = a-1;

4'b1101: c = b+1;

4'b1110: c = b-1;

4'b1111: c = b;

endcase

end

endmodule

Test Bench:

module ALU_tb();

reg [8:0] a;

reg [8:0] b;

reg [3:0] sel;

wire [8:0] c;

ALU uut (.c(c),.a(a),.b(b),.sel(sel));

initial

begin

a = 1;b = 1; sel = 4'b0000;

#10    a = 1;b = 0; sel = 4'b0001;

#10    a = 1;b = 1; sel = 4'b0010;

#10    a = 1;b = 1; sel = 4'b0011;

#10    a = 0;b = 1; sel = 4'b0100;

#10    a = 1;b = 0; sel = 4'b0101;

#10    a = 1;b = 1; sel = 4'b0110;

#10    a = 1;b = 1; sel = 4'b0111;

#10    a = 1;b = 1; sel = 4'b1000;

#10    a = 0;b = 1; sel = 4'b1001;

#10    a = 1;b = 0; sel = 4'b1010;

#10    a = 1;b = 1; sel = 4'b1011;

#10    a = 1;b = 0; sel = 4'b1100;

#10    a = 0;b = 0; sel = 4'b1101;

#10    a = 0;b = 1; sel = 4'b1110;

#10    a = 0;b = 0; sel = 4'b1111;

#10

$finish;

end

endmodule

RTL

Output